TOPIC: VERIFICATION OF A 5-STAGE PIPELINED RISC-V RV32IM PROCESSOR USING UVM

Members:

Huỳnh Dương Tôn Lực – 21161252

Ngô Quốc An – 21161226

Bùi Ngọc Duy Tân – 21161266

Objective:

- Survey RV32IM and test methodology (old and new) compared to UVM.

- Design RTL for RISC-V pipeline 5-stage.

- Build verification environment UVM (Universal Verification Methodology).

- Perform multi-level test: directed, random, ASM, compliance test.

- Evaluate coverage: functional coverage and code coverage.

Task allocation:

### System survey and specification

- Study RV32IM instruction set and define why we choose UVM for verification.

- Build pipeline data path: IF, ID, EX, MEM, WB.

- Design block: pipeline registers, register file, ALU, LSU, multiplier/divider, control unit.

- Handle hazard using stall & flush forwarding.

- Output: RTL core (Verilog).

### Verification environment design (System-Level Design)

- UVM driver/monitor for Instruction/Data bus.

- UVM sequencer/sequences: generate test stimulus (random/directed).

- UVM scoreboard: Compare output DUT with reference model.

- UVM coverage collector: collect coverage.

- Output: Complete UVM environment.

### Verification execution

- Directed test for ALU, branch, load/store, multiply/divide.

- Random test to improve (coverage).

- Compliance test using risc-v-arch-test for RV32IM.

- Collect bug log, waveform, and test report

### Coverage Analysis

- Functional coverage: monitor each instruction type and hazard.

- Code coverage: evaluate statement, branch, toggle, FSM coverage.

- Ensure coverage ≥ 90% for RV32IM.

### Report

- Write final report with coverage and waveform illustrations.

Expected result:

- 5-stage pipelined RV32IM RTL CPU runs all RV32IM subsets correctly.

- UVM environment automatically detects bugs and logs results.

- Pass compliance test (riscv-arch-test).

- Achieves coverage ≥ 90%.

Main test-case:

- Arithmetic & Logical (RV32I): ADD, SUB, AND, OR, XOR, SLT, SLTU

- Shift: SLL, SRL, SRA

- Immediate: ADDI, ANDI, ORI, XORI, SLTI, SLTIU, SLLI, SRLI, SRAI

- Load/Store: LW, SW, LH, LHU, SH, LB, LBU, SB

- Control Flow: BEQ, BNE, BLT, BLTU, BGE, BGEU, JAL, JALR

- Multiply & Divide (M-extension): MUL, MULH, MULHSU, MULHU, DIV, DIVU, REM, REMU

- Hazard: Data hazard → check stall, Control hazard → flush pipeline when branch taken, Compliance & Random

- Random test: mixed instruction sequences

Future Development:

- Add data forwarding to reduce stalls.

- Add branch predictor to reduce branch penalty.

- Extend ISA: F-extension (single precision float), C-extension (compressed).

- Integrate AXI4-Lite standard bus for SoC deployment.

- Advanced verification: constraint-random test, coverage-driven verification, formal verification (riscv-formal).

Application:

- Integrating 5-stage RV32IM pipeline CPU into SoC with standard bus (AXI4-Lite, Wishbone).

- Applications in IoT systems, controllers, and smart sensors.

- Use in wearable devices (smartwatches, fitness trackers) requiring low power consumption.

- RV32IM verification with UVM provides access to industry-standard chip design flows.

- Can be applied to ASIC tape-out process or FPGA implementation.

Keywords:

- RISC-V, RV32IM

- Pipeline Processor, 5-Stage Pipeline (IF, ID, EX, MEM, WB)

- Hazard (Stall, Flush, Forwarding, Branch Predictor)

- RTL Design (Verilog / System Verilog)

- UVM (Universal Verification Methodology)

- Directed Test / Random Test / Compliance Test

- Functional Coverage / Code Coverage

- Reference Model (ISS, Spike)

- Constraint-random Verification

- Coverage-driven Verification

- Formal Verification (risc-v-formal)

- SoC Integration (AXI4-Lite, Wishbone)

- Low-power Embedded Systems

References

1. Mallidu, J. and Siddamal, S.V., 2022. A survey on in-order 5-stage pipeline RISC-V processor implementation. In Information and Communication Technology for Competitive Strategies (ICTCS 2021) Intelligent Strategies for ICT (pp. 777-784). Singapore: Springer Nature Singapore.

2. Barbirotta, M., Mastrandrea, A., Menichelli, F., Vigli, F., Blasi, L., Cheikh, A., Sordillo, S., Di Gennaro, F. and Olivieri, M., 2020, October. Fault resilience analysis of a RISC-V microprocessor design through a dedicated UVM environment. In 2020 IEEE International Symposium on Defect and Fault Tolerance in VLSI and Nanotechnology Systems (DFT) (pp. 1-6). IEEE.

3. Wang, J., Tan, N., Zhou, Y., Li, T. and Xia, J., 2020, October. A UVM verification platform for RISC-V SoC from module to system level. In 2020 IEEE 5th International Conference on Integrated Circuits and Microsystems (ICICM) (pp. 242-246). IEEE.

4. Loh, S.H., Liew, Y.H. and Sim, J.J., 2022, October. VLSI Design Course with Verification of RISC-V Design using Universal Verification Methodology (UVM). In 2022 IEEE 12th International Conference on Control System, Computing and Engineering (ICCSCE) (pp. 7-12). IEEE.

5. Chen, J., Wang, Z., Ding, Y., Feng, H. and Chen, D., 2021, September. Design and verification of RISC-V CPU based on HLS and UVM. In 2021 IEEE International Conference on Computer Science, Electronic Information Engineering and Intelligent Control Technology (CEI) (pp. 659-664). IEEE.

6. Barraco, L., 2021. UVM Environment for RISC-V processor (Doctoral dissertation, Politecnico di Torino).

7. Budi, S., Gupta, P., Varghese, K. and Bharadwaj, A., 2018, March. A risc-v isa compatible processor ip for soc. In 2018 International Symposium on Devices, Circuits and Systems (ISDCS) (pp. 1-5). IEEE.

8. Kalmath, M., Kulkarni, A., Siddamal, S.V. and Mallidu, J., 2022. Implementation of 32-bit ISA five-stage pipeline RISC-V processor core. In Artificial Intelligence and Sustainable Computing: Proceedings of ICSISCET 2021 (pp. 243-251). Singapore: Springer Nature Singapore.

9. Zhao, X., Li, B., Yang, X., Wei, C., Xu, G., Bie, Z. and Jiang, K., 2025, July. Micro-architecture design based on reconfigurable AMBA bus. In Fourth International Conference on Electronics Technology and Artificial Intelligence (ETAI 2025) (Vol. 13692, pp. 388-394). SPIE.

10. Andorno, M., Andersen, M., Borghello, G., Caratelli, A., Ceresa, D., Dhaliwal, J., Kloukinas, K. and Pejasinovic, R., 2023. Rad-hard RISC-V SoC and ASIP ecosystems studies for high-energy physics applications. Journal of Instrumentation, 18(01), p.C01018.